Supplementary Information for

A Neuromorphic Single Pixel Compound Eye Breaks Temporal Resolution Limits for Wide-Field Motion Detection

Yu Cai1#, Guan Wang1#, Yihao Xue1, Huaxia Deng1,2*, Xinglong Gong1,3*

1CAS Key Laboratory of Mechanical Behavior and Design of Materials, Department of Modern Mechanics, University of Science and Technology of China, Hefei, Anhui 230027, China.

2State Key Laboratory of Nonlinear Mechanics, Institute of Mechanics, Chinese Academy of Sciences, 15 Beisihuan West Road, Beijing 100190, China.

3State Key Laboratory of Fire Science, University of Science and Technology of China, 96 Jinzhai Road, Hefei, Anhui 230026, China.

#: These authors’ contributions are equal.

*E-mails: hxdeng@ustc.edu.cn, gongxl@ustc.edu.cn.

This PDF file includes:

Supplementary Sections 1 to 8

Supplementary Figures 1 to 6

Supplementary Tables 1 to 4

Supplementary Video Legends

Supplementary Information References

Other Supplementary Materials for this manuscript include the following:

Supplementary Video 1 to 2

Supplementary Section 1: Detailed set up of the artificial ommatidium

The compound eye system comprises seven identical and individual bionic channels, with no optical crosstalk between neighboring optical channels, akin to the apposition compound eyes of insects. A comparison between our bionic channel and the natural insect ommatidium is shown in Supplementary Fig. 1. Each ommatidium typically includes a corneal lens, crystalline cone, rhabdom, pigment cell, photoreceptor cell, and optic nerve. The corneal lens and crystalline cone together form the dioptric apparatus, which gathers light and provides protective capabilities as the outermost layer of the ommatidium. In our system, we apply light cure adhesive to the front end of the optical fiber to serve the same roles of light gathering and protection. The optical fiber transmits the light collected by the dioptric apparatus, analogous to the function of the rhabdom, while the pigment cell and black polyethylene jacket ensure that light does not interfere between adjacent channels. Finally, the collected light is converted into electrical signals by the single-pixel detector, mimicking the photoreceptor cell function in natural ommatidia. The UV-cured adhesive has a refractive index between 1.49 and 1.7 and a light transmittance exceeding 90%, while optical fibers feature a core diameter of 8 mm and a numerical aperture of 0.5. These parameters ensure efficient light gathering and minimal signal loss, directly supporting the system's high signal fidelity.

Supplementary Fig. 1. The comparison between our bionic channel and the ommatidium of the apposition compound eye belonging to natural insects.

Supplementary Section 2: Detailed set up of the SPBACE

The experimental setup is depicted in Supplementary Fig. 2. The white light emitted from the light source (Luminus, CBT-140-White LEDs) illuminates a high-speed projection system, which includes a mirror, a digital micro-mirror device (DMD, Texas Instruments DLP Discovery 4100 development kit), and a camera lens. The DMD is the pivotal component in this arrangement. It functions to spatially mask light by adjusting its 1024×768 programmable micro-mirrors to ±12° orientations, corresponding to the “on” or “off” states, respectively1. The target and the background plate (black light-absorbing cloth) are positioned in front of the projection device. The black cloth mitigates reflections caused by structured illumination. Notably, the target is not placed on the focal plane. This setup demonstrates that even with the inherently blurry vision of insects, our proposed method can effectively locate the target. The modulated light projects marking patterns onto the target, and the backscattered or reflected light is collected by the compound eye system through seven bionic ommatidia. Each bionic ommatidium's photoelectric detector (PD, Thorlabs, PDA100C) converts the collected light intensity into corresponding voltage values. The PD has a low level of dark current, which is consistent with the typical performance of such silicon detectors optimized for general-purpose applications. The responsivity of PD exhibits a positive correlation with wavelength across the visible spectrum, rising from approximately 0.2 A/W at 400 nm to about 0.5 A/W at 700 nm.  The PD has a maximum response bandwidth 3 MHz and a noise equivalent power (NEP) range from 2.67×10⁻12 to 7.17×10⁻11 W/√Hz. These parameters enable efficient light gathering and minimal signal loss, thereby directly contributing to the system's performance in weak-light detection, high temporal resolution, and signal-to-noise ratio (SNR). The seven sets of voltage values are transmitted to a computer via a data acquisition system (DAQ, ART Technology USB3134A) for subsequent processing.

Supplementary Fig. 2. The diagram of the experimental setup.

The current prototype, which integrates seven ommatidia on a 3D-printed base, functions as an initial proof-of-concept to validate the fundamental principles of the SPBACE architecture. We recognize that the number of ommatidia in this implementation remains modest compared to natural compound eyes. A principal advantage of our design lies in its inherent scalability. The temporal resolution of the system is chiefly limited by the data acquisition hardware. In the present configuration employing a serial acquisition card, the total sampling rate is distributed among the active channels. Consequently, expanding the ommatidia count with existing hardware would proportionally lower the per-channel sampling rate, thereby reducing the temporal resolution. This constraint, however, is not intrinsic to the SPBACE architecture. The temporal resolution could be preserved independent of the number of ommatidia by implementing a data acquisition system capable of simultaneous parallel sampling across all channels. Such an advancement would enable the system to accommodate a substantially larger array of ommatidia while retaining the high-speed detection performance demonstrated herein. It should be noted that a primary trade-off emerges with the increase in ommatidia count: a linear rise in total data throughput. Regarding miniaturization and power consumption, the SPBACE architecture exhibits considerable potential. Its core components (optical fibers and single-pixel photodetectors) are inherently compact. Future iterations could employ advanced micro-fabrication techniques, such as the direct integration of microfiber bundles with miniature photodetector arrays on a unified substrate, to realize a significantly reduced form factor that more closely approximates biological systems. In summary, achieving a higher-density, miniaturized SPBACE is feasible through a synergistic combination of parallel data acquisition and advanced micro-optics integration, making it suitable for applications where size, weight, and power are critical constraints.

Supplementary Section 3: The improvement of the reconstructed image in Hadamard single pixel imaging experiment

Supplementary Fig. 3. Results of Hadamard single pixel imaging experiment. Imaging reconstructed from seven individual ommatidia channels and the superimposing channel.

The proposed computational approach is not limited to one single-pixel imaging algorithm but represents a general framework applicable to various single-pixel modalities. As shown in Supplementary Fig. 3, we validated the method using Hadamard single-pixel imaging, where multi-channel signal fusion similarly enhanced image quality by reducing noise and improving reconstruction fidelity. This universality stems from the core principle of processing one-dimensional signals from multiple photodetectors.

Supplementary Section 4: The principles and process of locating

The process of locating the target is composed of two steps. Firstly, the Fourier coefficients of the target sense are acquired by single-pixel imaging (SPI) technology. Unlike the conventional imaging method which employs image sensors like charge-coupled device (CCD) or complementary metal oxide semiconductor (CMOS), SPI allows us to utilize a single-pixel photoelectric detector (PD) with no spatial resolution to obtain the spatial information of objects and reconstruct the image through various algorithms2. The core technology of it is encoding the target with the modulation light, and then the 1D electric signal is decoded to reconstruct the image. With a specific frequency pair (fx,fy) and an arbitrary initial phase φ, a Fourier modulation pattern P can be expressed as:

Px,y fx,fy,φ)=A+B∙cos⁡[2πfx , fy+φ]                                 (S1)

where x and y are the position coordinates in the 2-D spatial domain, A denotes the average of the pattern and B represents the contrast. The formula of the light intensity collected by the single-pixel PD is shown as equation S2.

Vfx,fy,φ=M∙  R(x,y)∙Px,y fx,fy,φ)dxdy+Dn                        (S2)

R(x,y) is the intensity distribution of the image, M represents the scale factor which is related to the environment and type of the photodetector, and Dn is the noise.

According to the formulas mentioned above, for a 2D image, the information acquired by a single-pixel PD is always identical no matter where the photodetector is. In the previous experiment, for a 3D object, its spatial form can be captured by using several single-pixel PDs in different locations3. Here, the different Fourier coefficients about the 3D ball are obtained from the seven single-pixel PDs as the seven bionic channels in diverse positions. Therefore, the light intensity collected by the i-th PD can be expressed as:

Vifx, fy,φ=M∙  R(x,y)∙Px,y fx,fy, φ)dix,y∙nx,ydxdy+Dni          (S3)

where R(x ,y) is the surface reflectivity of the 3D object, di is the unit vector pointing from the pixel x,y on the object to the lens of i-th bionic channel, and nx,y is the surface normal unit vector for the object.

Then the complex Fourier coefficient Cifx , fy of i-th bionic channel can be acquired in manner of the 3-step phase-shifting method through the data of corresponding PD. These three Fourier patterns have the same frequency pair (fx , fy), while the initial phases φ are 0, 2π3, 4π3, respectively. The formula of Cifx , fy can be expressed as:

Cifx , fy=232D0i-D2π3i-D4π3i+3j∙D2π3i-D4π3i                         (S4)

where j represents the imaginary part. Through the differential measurements, the average A and the noise Dn can be eliminated. Therefore, it can directly extract the coefficients of specific frequency pair (fx , fy) in the Fourier frequency domain without reconstructing the image.

Because most of the energy of images is concentrated in the low-frequency of Fourier spectrum, we select the central 11×11 pixels of Fourier coefficients as the input of the Bionics Multi-channel Optic Lobe Network (BMOLN). According to the central symmetry characteristic of Fourier spectral coefficients, we only need to collect half of them, that is, 61 Fourier coefficients. Therefore, totally 183 patterns are used to acquire 121 Fourier coefficients of the central area through 3-step phase-shifting method. Each projection of 183 patterns can be regarded as a single information acquisition of compound eye structure. The original FSPI uses gray-scale Fourier basis patterns for illumination. Because of its sinusoidal intensity of pattern, the refreshing rate of DMD is 250 Hz, while the maximum refreshing rate of DMD can reach 22,000 Hz for binary modulation. In order to improve the efficiency, the Floyd-Steinberg error diffusion dithering method4 is utilized to obtain the binary Fourier patterns. It has been calculated that the max theoretical temporal resolution of proposed compound eye is about 120 signals per second, which is close to the 100 signals per seconds of the honeybee with apposition compound eye5. The DAQ operating at 250KHz is employed to digitalize the single-pixel measurements. However, the DQA used in this work is asynchronous acquisition, the acquisition rate needs to be divided by 7, that is about 35.7KHz. And limited by the balance of projection refreshing rate and the quality of collected signal, the frame rate of our compound eye system only reaches 13 fps.

The second step is that the Fourier coefficients are sent into the BMOLN as the input, which subsequently outputs the locating information about the target. Before the BMOLN can achieve the locating task, it needs to be calibrated trained. Each bionic channel has its network. Therefore, the network in the same framework is trained with seven groups of different data. Seven frames with the same parameters and different networks are used to obtain the location information of the corresponding bionic channels. According to biological theory, the visual nervous system of insects is far inferior to that of humans in terms of both the number and function of nerve cells6. Therefore, we designed a BMOLN with a simple structure to simulate the visual signal system of insects at a high location detection speed, although a sophisticated network framework may get better results. It needs to be emphasized that the Fourier coefficient is a complex number, so we take the value of the real part and the value of the imaginary part as the input of CNN, respectively. The convolution operation is used to extract the features of the input data. And the feature map of l layer can be expressed as:

Hl=fWlHl-1+bl                                                       (S5)

In formula (5), Wi represents the weight of the convolution kernel in l layer. Wi is convolved with (‘’) the feature map about l-1 layer, and then it is added with the bias term of the l layer (bl). Finally, the feature map of current layer (Hl) is acquired by the nonlinear function (fx), which is the ReLU (Rectified Linear Unit) in this work. The Squeeze-and-Excitation (SE) block is introduced to assign different weight coefficients for each channel, which can explicitly model the interdependencies between convolutional feature channels to improve the representation capability of the network7.

The BMOLN architecture is designed to structurally mimic the lamina-medulla-lobula neuropil of Drosophila. As illustrated in Supplementary Fig. 4, the network comprises an input layer accepting 11×11×2 feature maps (real and imaginary parts of Fourier coefficients), followed by three 2D convolutional layers with 32, 64, and 64 filters respectively. Each convolutional layer employs 3×3 kernels, and same padding to maintain spatial dimensions. A Squeeze-and-Excitation (SE) attention mechanism is integrated after each convolution to adaptively recalibrate channel-wise feature responses. The resulting features are flattened and processed through three fully connected layers (256, 128, and 2 neurons) to output spatial coordinates of the x axis and the y axis. For the training process, the loss function can be seen in equation S6.

MSE=1mi=1mxci-xti2+yci-yti2                            (S6)

where xc and yc are the output of the BMOLN, while the xt and yt are true location of the target. And the optimal weight can be acquired through minimize the loss function.

Supplementary Fig. 4. The architecture of the proposed network.

In the locating experiments, a white ball with a diameter of 7 mm is used as the target object. The resolution of Fourier patterns is 64×64 pixels, and the size of projection plane is 132 mm×132 mm.   The training procedure utilized a dataset of 2007 samples, with each sample representing a distinct static position of the target under uniform illumination conditions. The BMOLN was trained to learn a direct correspondence between the measured Fourier coefficients and the spatial coordinates of the target. Since the network acquires knowledge from the fundamental optical characteristics of a stationary scene, the established mapping remains unaffected by the target's dynamic state. This inherent property endows the network with robust generalization abilities for tracking moving objects. The training employed a root mean square error (RMSE) loss function to minimize the discrepancy between predicted and actual coordinates. Optimization was carried out over 60 epochs with a batch size of 32, while 10% of the data was allocated for validation and evaluated at 15-epoch intervals to prevent overfitting.

After obtaining 7 bionic channels’ positions of the target, the Kalman algorithm is used to fusion multiple positions to improve the accuracy and stability. The fusion strategy relies on the estimated measurement error associated with each channel, rather than adapting to specific motion patterns. The Kalman gain parameter is calculated for each channel based on the maximum localization error observed in the validation dataset. This methodology assigns different weights to the all ommatidia, giving more influence to channels with superior inherent accuracy. As a result, the fused output achieves enhanced localization precision and stability across varying motion conditions without necessitating real-time parameter adjustments. The Kalman algorithm is divided into three steps. Firstly, the fusion gain coefficient Fi of i-th channel is calculated as:

Fi=eESTi-1eESTi-1+eMEAi                                                           (S7)

where eESTi-1 is estimated error of (i-1)-th channel, and eMEAi is measurement error of i-th channel. The measurement error of each channel is set as the maximum error of validation set. Then the estimated value xi of  i-th channel as:

xi = xi-1+Fizi-xi-1                                                    (S8)

Where zi is the measurement value of i-th channel. Finally,  eMEAi is updated as:

eESTi=1-FieESTi-1                                                     (S9)

Repeat the process until i=7. We start this process as i=2. We set eEST1=eMEA1.

Supplementary Table 1: The locating errors of the static object

|  | Mean locating error |  | Maximum locating error |  |
| --- | --- | --- | --- | --- |
| Bionic channel number | X axis (pixel) | Y axis (pixel) | X axis (pixel) | Y axis (pixel) |
| 1 | 0.7921 | 0.7485 | 2.0976 | 3.0107 |
| 2 | 0.9128 | 0.6263 | 2.0723 | 1.4565 |
| 3 | 0.5526 | 0.8004 | 1.7107 | 2.1424 |
| 4 | 1.2519 | 1.1389 | 5.8519 | 3.4896 |
| 5 | 0.8785 | 0.9043 | 2.5639 | 2.1533 |
| 6 | 0.9484 | 0.9092 | 2.5790 | 2.3920 |
| 7 | 0.9126 | 0.8170 | 2.8173 | 3.4211 |
| 1+2+4 | 0.6148 | 0.5108 | 1.9893 | 1.3778 |
| 3+4+5 | 0.4133 | 0.5117 | 1.0578 | 1.7960 |
| 5+6+7 | 0.6729 | 0.7672 | 1.9106 | 2.0054 |
| 1+2+3+4+5 | 0.3989 | 0.4032 | 1.0882 | 1.2699 |
| 1+2+3+6+7 | 0.3155 | 0.4788 | 0.8265 | 1.4580 |
| 3+4+5+6+7 | 0.5034 | 0.5756 | 1.3041 | 1.6819 |
| 1+2+3+4+5+6+7 | 0.3264 | 0.4319 | 0.7649 | 1.2902 |

Supplementary Section 5: Acquiring the accurate positions of the moving target

The high brightness of the modulated patterns makes it impossible for the conventional camera to capture the accurate positions of the moving white ball, which is shown in Supplementary Video 2. Thus, a computational method is proposed to measure the positions of the object in pendulum motion. According to the measurement, the area of the projected pattern is 132 mm × 132 mm, the transparent rope's length is 240 mm, and the rope's length within the projected area is 105 mm. Furthermore, the distance from the left edge of the projection to the target is 64 mm. Given that the units for projected patterns (64 × 64 pixels) and locating coordinates are in pixels, these parameters are proportionally converted to pixel lengths. The converted values are illustrated in Supplementary Fig. 5. The formula for calculating the two-dimensional coordinates is expressed as:

y=1162-x-312-65                      (S10)

where x and y denotes the 2-D coordinate on the projected plane. The 30 x coordinates obtained by the network and fusion are substituted into Equation (S10) to acquire the corresponding 30 y coordinates, acquiring the ground truth of the moving target.

Supplementary Fig. 5. The diagram of calculating the pendulum trajectory.

Supplementary Table 2: The locating errors of the moving object

| Bionic channel number | Mean locating error (pixel) | Maximum locating error (pixel) |
| --- | --- | --- |
| 1 | 0.9359 | 2.0007 |
| 2 | 0.7917 | 1.8103 |
| 3 | 0.9193 | 3.2178 |
| 4 | 0.8318 | 2.5559 |
| 5 | 0.6354 | 1.6515 |
| 6 | 0.8659 | 2.1543 |
| 7 | 0.9436 | 2.1427 |
| 1+2+4 | 0.4740 | 1.2250 |
| 3+4+5 | 0.5585 | 2.0304 |
| 5+6+7 | 0.7124 | 2.0903 |
| 1+2+3+4+5 | 0.5836 | 2.1146 |
| 1+2+3+6+7 | 0.6505 | 2.0514 |
| 3+4+5+6+7 | 0.5116 | 1.9583 |
| 1+2+3+4+5+6+7 | 0.5364 | 1.5381 |

Supplementary Section 6: The positive relationship between the number of ommatidia and the vision ability of insects

Based on the experimental results, it’s concluded that as the number of ommatidia increases, visual ability such as locating accuracy and motion sensitivity improves gradually. This positive correlation between visual ability and the number of ommatidia channels of the SPBACE provides new evidence to support a previously unproven hypothesis. This hypothesis suggests that the visual ability of the natural compound eye is related to the number of ommatidia, based on observations in nature. Despite being of the same species, insects in different roles or habitats exhibit variations in the number of ommatidia. As shown in Supplementary Table S3, the average numbers of ommatidia of the 2 species of ants but with different roles are displayed. Ants dedicated to reproduction have the largest number of ommatidia, while major workers have more ommatidia than minor workers. Reproductive ants, responsible for mating flights, have the highest requirements for visual acuity and sensitivity. Major workers, responsible for foraging, require higher sensitivity and locating accuracy than those involved in nest construction. Overall, flying ants have significantly more ommatidia than crawling ants, as high-speed flight demands greater motion sensitivity than low-speed crawling. In addition, the number of ommatidia in the same insects varies significantly depending on their habitat, as shown in Supplementary Table S3. Due to complex ambient light, intricate scenes, and a greater presence of predators such as dragonflies and frogs, outdoor terrain necessitates a higher level of visual acuity and accuracy. Therefore, outdoor flies such as blowflies and flesh flies exhibit significantly higher numbers of ommatidia than indoor houseflies, facilitating enhanced visual sensitivity and spatial acuity. Our results demonstrate that an increase in the number of ommatidia improves the capacity for locating and motion sensitivity, supporting Darwin's theory of evolution.

Supplementary Table 3. Average number of ommatidia of different insects per site

| Genus | Species | Main labor or main living environments | Ommatidia Number |
| --- | --- | --- | --- |
| Ant | Solenopsis invicta8 | Nest construction (Minor worker) | 48 |
|  |  | Foraging (Major worker) | 93 |
|  |  | Reproduction (Queens) | 589 |
|  |  | Reproduction (Males) | 870 |
|  | Camponotus pennsylvanicus9 | Nest construction (Minor worker) | 375 |
|  |  | Foraging (Major worker) | 658 |
|  |  | Reproduction (female) | 1023 |
|  |  | Reproduction (male) | 1036 |
| Fly | Musca domestica10 | Indoor | 3484 |
|  | Liosarcophaga dux10 | Outdoor | 6032 |

Supplementary Fig. 6. Collision experiment results. (a) Schematic diagram of the collision experiment. (b) The front view of the compound eye structure. (c)-(i) The measured voltage values of no object (red lines) and collision (blue lines).

Supplementary Section 7: The criteria for the triggering mechanism of collision alarm in NLOS scenes

The maximum acquisition rate of our data acquisition system is 250 kSps (kilosamples per second) using an asynchronous acquisition method. Consequently, the maximum acquisition rate of the seven-channel SPBACE system is approximately 35.7 kSps. In the non-line-of-sight (NLOS) experiment, the addition of an eighth channel, which serves as the reference system, reduces the maximum acquisition rate to 31.3 kSps.

When objects reside outside the direct field of view, light undergoes ​​multipath reflections​​ before detection by the SPBACE system. ​​Coupled with source flickering effects of the light source​​, fluctuating voltages are measured, ​compromising the reliability of the 3σ criterion​​ for NLOS collision detection. To solve this problem, we refined the criteria for the triggering mechanism, as collision alerts are activated only if ​​at least three out of five consecutive measurements​​ deviate beyond the 3σ threshold. Using this method, bionic channel 1 detects that the onset of data indicating an object entering the non-visual field occurs 614 data points later than in the reference system, corresponding to a delay time of 20 ms. Under real-world outdoor illumination, ambient light fluctuations are substantially more pronounced. Consequently, for outdoor experiments, the decision criterion was further tightened. A collision alert is triggered only when five consecutive measurements deviate beyond the 3σ threshold.

Supplementary Section 8: Calculation and comparison of temporal resolution

The unit "ss/s" (signals per second) employed in this work to express temporal resolution is defined as the number of distinguishable signals per unit time for each single-pixel photodetector (PD). Here, one signal corresponds to the measurement required to obtain one unit of effective information. For example, in conventional imaging approaches, information is generally derived from complete image data. If a particular task necessitates capturing two successive images to acquire the required information, the temporal resolution for that task equates to the camera frame rate (for example 30 fps) divided by the number of images required (assuming as 2 pieces), resulting in a value of 15 ss/s. Conversely, in the SPBACE system, collision detection is achieved by assessing whether an individual light intensity measurement lies within the interval of the mean value plus or minus three standard deviations. Accordingly, the temporal resolution is derived by dividing the sampling rate of each channel, 35,174 kSamples per second (kSps), by the number of necessary light intensity values, which is one, yielding 36,714 ss/s. In essence, temporal resolution fundamentally reflects the hardware data acquisition capacity and the number of measurements essential for retrieving one information unit, independent of response latency and signal processing delays. The achievement of ultra-high temporal resolution stems from both the high-speed data acquisition capability of the hardware and the processing approach of the bionic architecture, which requires only a single measurement.

A direct comparison between "ss/s" and "fps" is valid, as both metrics quantify the frequency of fundamental information acquisition events. Conventional camera systems typically require at least one complete image frame to extract usable information, and their achievable temporal resolution is inherently limited by the frame rate of the sensor array. In contrast, the biomimetic design of our system enables direct collision detection through light intensity measurements from a single photodetector, eliminating the need for computational image reconstruction. A comparison of the SPBACE with both commercially available artificial compound eye systems and natural insects, as presented in Table 4, clearly demonstrates the orders-of-magnitude advantage of the single-pixel, non-imaging scheme in temporal performance. While the temporal resolution of our system is 1152 times higher than that of a specific commercially available compound eye system, a conservative factor of approximately 600 is reported in the main text, considering the prevalence of highly mature commercial CMOS and CCD systems operating at 60 fps.

Supplementary Table 4. Comparing temporal resolution across commercial compound eyes, biological systems, and the SPBACE.

| Type | Source | Temporal resolution |
| --- | --- | --- |
| Commercial compound eye based on CCD/COMS | OMAT (OmniMatrix) | 25 fps |
|  | TY-CE (Magixoom) | 30 fps |
|  | VA6H (VOMMA) | 31 fps |
| Natural insect | Ref. 11 | 300 ss/s |
| SPBACE | Our work | 35714 ss/s |

Supplementary Video Legends

Supplementary Video 1 (separate file). Demonstration of locating the static ball.

Supplementary Video 2 (separate file). Demonstration of locating the moving ball.

Supplementary Information References

1.	Gibson, G. M., Johnson, S. D. & Padgett, M. J. Single-pixel imaging 12 years on: a review. Opt. Express. 28, 28190 (2020).

2.	Ni, M., Cai, Y., Deng, H. & Gong, X. An adaptive and dynamic lens-flare-free single-pixel imaging method for spacecraft docking. Opt. Laser. Eng. 161, 107391 (2023).

3.	Sun, B. et al. 3D Computational Imaging with Single-Pixel Detectors. Science. 340, 844-847 (2013).

4.	Peng, H. et al. Ringing-free fast Fourier single-pixel imaging. Opt. Lett. 47, 1017 (2022).

5.	Wu, S. et al. Artificial compound eye: a survey of the state-of-the-art. Artif Intell Rev. 48, 573-603 (2017).

6.	de Croon, G. C. H. E., Dupeyroux, J. J. G., Fuller, S. B. & Marshall, J. A. R. Insect-inspired AI for autonomous robots. Sci. Robot. 7, eabl6334 (2022).

7. Hu, J., Shen, L., & Sun, G.. Squeeze-and-excitation networks. Proc. IEEE Conf. Comput. Vis. Pattern Recog. 7132-7141, (2018).

8. Baker, G. T. & Ma, P. W. K. Morphology and number of Ommatidia in the compound eyes of Solenopsis invicta, Solenopsis richteri, and their hybrid (Hymenoptera: Formicidae). Zoologischer Anzeiger. 245, 121-125 (2006).

9. Klotz, J. H., Reid, B. L. & Gordon, W. C. Variation of ommatidia number as a function of worker size in Camponotus pennsylvanicus (DeGeer) (Hymenoptera: Formieidae). Ins. Soc. 39, 233-236 (1992).

10. Sukontason, K. L. et al. Ommatidia of blow fly, house fly, and flesh fly: implication of their vision efficiency. Parasitol. Res. 103, 123-131 (2008).

11. Wu, S. et al. Artificial compound eye: a survey of the state-of-the-art. Artif. Intell. Rev. 48, 573-603 (2017).